Exp No : 15

SERIAL INTERRUPT USING UART (KEYBOARD INPUT) IN KEIL

AIM : To write and simulate an Embedded C program using serial interrupt in 8051 microcontroller such that while a message is continuously transmitted, any key pressed from the keyboard generates an interrupt and is displayed through UART.

SOFTWARE REQUIRED :

- PC with Windows OS

- Keil μVision IDE

- 8051 Simulator (Keil built-in)

- Serial Window (UART #1)

PROGRAM :

#include <reg51.h>

#include <stdio.h>

void serial_ISR(void) interrupt 4

{

char ch;

if (RI)

{

RI = 0;        // Clear receive interrupt flag

ch = SBUF;     // Read received character

printf("\n>>> INTERRUPT RECEIVED: ");

printf("%c\n", ch);

}

}

void delay(void)

{

unsigned int i;

for(i = 0; i < 50000; i++); // Delay for visibility

}

void main(void)

{

SCON = 0x50; // UART mode 1, 8-bit, REN enabled

TMOD = 0x20; // Timer1 mode 2 (auto-reload)

TH1  = 0xFD; // Baud rate 9600

TR1  = 1;    // Start Timer1

IE   = 0x90; // EA = 1, ES = 1 (enable serial interrupt)

TI   = 1;    // Enable printf

while(1)

{

printf("Hello World\n");

delay();

}

}

Circuit Diagram:

RESULT :

Thus, the serial interrupt using UART was successfully implemented and verified in Keil simulator. The interrupt was triggered upon keyboard input and serviced correctly using an Interrupt Service Routine.