Exp No : 5

ARDUINO BASED MANUAL ELECTRONIC COUNTER USING PROTEUS

AIM :

To write an Embedded C Program for ARDUINO BASED MANUAL ELECTRONIC COUNTER USING PROTEUS

SOFTWARE REQUIRED :

- Proteus 8 software

- Arduino IDE software

PROGRAM :

int x0,x1,x2,x3,x4,x5,x6,x7,x8,x9;

int delay_time = 200;

void setup() {

// configure pin12 as an input and enable the internal pull-up resistor

pinMode(12, INPUT_PULLUP);

pinMode(1, OUTPUT);

pinMode(2, OUTPUT);

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

pinMode(5, OUTPUT);

pinMode(6, OUTPUT);

pinMode(7, OUTPUT);

}

void loop() {

// read the pushbutton value into a variable

int sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x0 = true;

}

while (x0) {

zero();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x1 = true;

x0 = false;

}

}

while (x1) {

one();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x2 = true;

x1 = false;

}

}

while (x2) {

two();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x3 = true;

x2 = false;

}

}

while (x3) {

three();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x4 = true;

x3 = false;

}

}

while (x4) {

four();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x5 = true;

x4 = false;

}

}

while (x5) {

five();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x6 = true;

x5 = false;

}

}

while (x6) {

six();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x7 = true;

x6 = false;

}

}

while (x7) {

seven();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x8 = true;

x7 = false;

}

}

while (x8) {

eight();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x9 = true;

x8 = false;

}

}

while (x9) {

nine();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x0 = true;

x9 = false;

}

}

}

void zero() {

for (int i = 1; i < 7; i++) {

digitalWrite(i, HIGH);

digitalWrite(7, LOW);

}

delay(delay_time);

}

void one() {

digitalWrite(1, LOW);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

delay(delay_time);

}

void two() {

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, LOW);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, LOW);

digitalWrite(7, HIGH);

delay(delay_time);

}

void three() {

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, HIGH);

delay(delay_time);

}

void four() {

digitalWrite(1, LOW);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

void five() {

digitalWrite(1, HIGH);

digitalWrite(2, LOW);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, LOW);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

void six() {

digitalWrite(1, HIGH);

digitalWrite(2, LOW);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

void seven() {

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, LOW);

digitalWrite(5, LOW);

digitalWrite(6, LOW);

digitalWrite(7, LOW);

delay(delay_time);

}

void eight() {

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, HIGH);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

void nine() {

digitalWrite(1, HIGH);

digitalWrite(2, HIGH);

digitalWrite(3, HIGH);

digitalWrite(4, HIGH);

digitalWrite(5, LOW);

digitalWrite(6, HIGH);

digitalWrite(7, HIGH);

delay(delay_time);

}

CIRCUIT DIAGRAM :

RESULT :

Thus the program has been successfully verified and executed.